- This STATEMENT confirms that involved in a process technical personnel well familiar with the technical documentation used during maintenance process and well recognize that this job card may be executed in a not strict order of step-by-step sequence. This shift of technological operations will not result on component condition at the moment of release to service and will not affect its airworthiness.

- Obey all warnings, cautions & notes listed in CMM/SPM sections that are in use per CMJC items.

- Use tool and equipment (including alternative) IAW “Перечень АТИ ЦТОК”.

| - ACCOMPLISHMENT INSTRUCTIONS 1.1 PREPARATION |  |  |  |
| --- | --- | --- | --- |
| 1.1.1 | Write down used Measurement Tools and Torque Tools. Description; ID Number. Digital multimeter (Fluke 87). Digital inclinometer (RGK U-66). Milliohmmeter (AKIP-6301). Megohmmeter (GPT-79603). Stopwatch. Digital caliper (SCC-I-250-0.01). Digital scales (PVM-3/30-P). Torque Range; ID Number. 0.5 - 2.5 Nm (A.402). 2 - 10 Nm (A.404). Ref. Перечень АТИ ЦТОК. | QM / CCS |  |
| 1.1.2 | Perform identify possible corrective actions based on CMJC 01-334212-XX test results (if applicable) or all available information on the malfunction and Fault Isolation Table. Ref. CMM pages 1011-1012, item 2. | QM / CCS |  |
| 1.1.3 | Perform identify implemented SB’s based on Identification Label and Service Bulletin List. Ref. CMM pages SBL 1,2. | QM / CCS |  |
| 1.2 DISASSEMBLY |  |  |  |
| 1.2.1 | Perform Disassembly. ATTENTION: Do NOT do more disassembly that are necessary. Ref. CMM pages 3001-3005 item 1.B. | QM / CCS |  |
| 1.3 CLEANING |  |  |  |
| 1.3.1 | Perform Cleaning. - Clean the non-electrical parts with cleaning solvent and dry with compressed air. - Clean the electrical parts and the motor with a clean, dry, lint free cloth. ATTENTION: Do NOT clean the sleeve bearings or the assemblies containing sleeve bearings with cleaning solvent. Clean these parts with a clean, dry, lint free cloth. Ref. CMM page 4002 item 1.C. | QM / CCS |  |

| 1.4 INSPECTION / CHECK ATTENTION: All replaced parts must be recorded in the Item Change List. ATTENTION: After replacing the part, repeat the corresponding step of the CMJC. |  |  |  |
| --- | --- | --- | --- |
| 1.4.1 | Perform Visual Check. - Check for scratches and shock marks, for good condition of the connecting parts including the electrical connector, for good condition of the anti-corrosion protections, for absence of external scratches on the lamp glass, for correct wire-locking of all screws and for presence of the filament positioning label. - Check condition of the gear sector, of the teeth of the screw, the wheels and the drive gear of the motor, of the mobile lamp housing, of the main casing, of the clamp assembly and of the motor casing. Ref. CMM pages 5001-5002 item 1.C(1). | QM / CCS |  |
| 1.4.2 | Perform Motor Check. - Perform visual check of all parts. - Make sure there are no nicks, dents or other deformations. Ref. CMM page 5002 item 1.C(2(a-c)). | QM / CCS |  |
| 1.4.3 | Perform Electrical Check of the Fixed Capacitor. Ref. CMM page 5002 item 1.C(3(a)). | QM / CCS |  |
| 1.4.4 | Perform Electrical Check of the Motor Assembly. - With a Megohmmeter check of the electrical insulation. The insulation resistance must be greater than 20 MΩ below 500 VDC. - Make sure that the motor starts with an AC voltage supply of 115 V / 400 Hz. Ref. CMM page 5002 item 1.C(3(b)). | QM / CCS |  |
| 1.4.5 | Perform Electrical Check of the Electromagnetic Relay. - Clean the contacts. - Make sure that there is no strong sparking during electrical testing. - With a Megohmmeter check of the electrical insulation. The insulation resistance must be greater than 20 MΩ below 500 VDC. - Make sure that relay operate with an AC voltage supply of 115 V / 400 Hz. Ref. CMM page 5003 item 1.C(3(c)). | QM / CCS |  |
| 1.4.6 | Perform Electrical Check of the Sensitive Switches. - Perform visual check of the leafspring. - With a Megohmmeter check of the electrical insulation. The insulation resistance must be greater than 100 MΩ below 500 VDC. - Make sure the sensitive switches operate correctly when you operate their leafspring. When the contact is switched ON or OFF, cut-off must be sharp. - Energize the contacts with a current of 2.5 A at 115 V / 400 Hz and make sure the cut-off is sharp. Ref. CMM page 5003 item 1.C(3(d)). | QM / CCS |  |
| 1.4.7 | Perform Electrical Check of the Auto-Transformer. - Perform visual check of primary and secondary terminal for signs of overheating. - With a Megohmmeter check of the electrical insulation. The insulation resistance must be greater than 100 MΩ below 500 VDC. - Make sure the circuits of the primary and secondary windings are continuous. Ref. CMM page 5003 item 1.C(3(e)). | QM / CCS |  |
| 1.4.8 | Perform Dimensional Check. □ Performed □ N/R NOTE: Perform this check only if incorrect dimensions are suspected. Ref. CMM page 5003 item 1.C(4), page 9 figure 2. | QM / CCS |  |
| 1.4.9 | Perform Weight Check. □ Performed □ N/R NOTE: Perform this check only if incorrect weight is suspected. Ref. CMM page 5003 item 1.C(5), page 1 item 1.B(2). | QM / CCS |  |
| 1.4.10 | Certify that all CMJC instructions related to ”INSPECTION / CHECK” have been completed in accordance with the CMM. | CCS |  |

| 1.5 REPAIR ATTENTION: All used consumable materials and spare parts must be recorded in the Material Used Table and Item Change List. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.5.1 | Perform Repair of the Mechanical Parts. - Use colorless aluminizing with a soft brush for protection of the mobile lamp housing. - Use chemical nickel plating with a soft brush for protection of the lamp clamps. □ Performed □ N/R Ref. CMM page 6002 item 1.D(1). |  |  |  | QM / CCS |  |  |
| 1.5.2 | Perform Repair of the Receptacle Connector. □ Performed □ N/R Ref. CMM pages 6002-6003 item 1.D(2). |  |  |  | QM / CCS |  |  |
| 1.5.3 | Certify that all CMJC instructions related to ”REPAIR” have been completed in accordance with the CMM. |  |  |  | CCS |  |  |
| 1.6 ASSEMBLY |  |  |  |  |  |  |  |
| 1.6.1 | Perform Assembly. NOTE: For required torque values see “FITS AND CLEARANCES” section. NOTE: For the adjustment of the "EXTENSION" or "RETRACTION" position use Test Set TD01 for light installation (make sure that Test Set is level) and Functional Tester RLTS01-AT for electrical power connection (use “S3” switch for extension and “S4” switch for retraction of the light). Ref. CMM pages 7002-7016 item 1.D, page 8001. |  |  |  | QM / CCS |  |  |
| 1.6.2 | Certify that all CMJC instructions related to ”ASSEMBLY” have been completed in accordance with the CMM. |  |  |  | CCS |  |  |
| 1.7 TESTING ATTENTION: All test results must be recorded in the Test / Check Results Table. NOTE: The required values / states and their limits are specified in the Test / Check Results Table. |  |  |  |  |  |  |  |
| 1.7.1 | Make sure that the ambient conditions are suitable for testing. Description; Limit; Acceptability. Temperature; 25 ± 10 °C; □ Ok □ NOT Ok. Relative humidity; Less than 85 %. Atmospheric pressure; 84 to107 kPa. Ref. CMM page 1002, item 1.B(3). | QM / CCS |  |  |
| 1.7.2 | Perform Bonding Test. With a Milliohmmeter measure the resistance between the connector pin “J” and the ground studs. Ref. CMM page 1002, item 1.D. |  |  |  | QM / CCS |  |  |
| 1.7.3 | Perform Insulation Test. For P/N’s 4191695 / 4315542 / 4331772. Connect all the connectors pins together except “J” and apply 500 VDC with a Megohmmeter between all the connected pins and the ground studs. For P/N 117PE01Y00. Connect all the connectors pins (A-B-C-F-H-K-L) together and apply 500 VDC with a Megohmmeter between all the connected pins and the casing of the light. Ref. CMM page 1002, item 1.E. |  |  |  | QM / CCS |  |  |
| 1.7.4 | Perform Extension Test. - Connect light to the Functional Tester RLTS01-AT. Make sure that the switches “S1”, “S2”, “S3” and “S4” in the “OFF” position. - Set the switch “S3” to "ON" position. Wait full extension of the light. - After end of extension set the switch “S3” to the "OFF" position. Ref. CMM page 1004, item 1.F. |  |  |  | QM / CCS |  |  |
| 1.7.5 | Perform Retraction Test. - Set the switch “S4” to the "ON" position. Wait full retraction of the light. - After end of retraction set the switch “S4” to the "OFF" position. Ref. CMM page 1004, item 1.G. |  |  |  | QM / CCS |  |  |
| 1.7.6 | Perform Relay Test. - Set the switch “S2” and “S3” to the "ON" position. Wait full extension of the light. - After end of extension set the switch “S3” to the "OFF" position. - Set the switch “S4” to the "ON" position. Wait full retraction of the light. - After end of retraction set the switch “S2” and “S4” to the "OFF" position. Ref. CMM page 1005, item 1.H. |  |  |  | QM / CCS |  |  |
| 1.7.7 | Perform Lamp Test. - Set the switch “S1”, “S2” and “S3” to the "ON" position. Wait full extension of the light. - After end of extension set the switch “S3” to the "OFF" position. - Set the switch “S4” to the "ON" position. Wait full retraction of the light. - After end of retraction set the switch “S1”, “S2” and “S4” to the "OFF" position. Ref. CMM pages 1005-1006, item 1.I. |  |  |  | QM / CCS |  |  |
| 1.7.8 | Perform Indication Switch Test. - Set the switch “S3” to "ON" position. Wait full extension of the light. - After end of extension set the switch “S3” to the "OFF" position. - Set the switch “S4” to the "ON" position. Wait full retraction of the light. - After end of retraction set the switch “S4” to the "OFF" position. Ref. CMM page 1006, item 1.J. |  |  |  | QM / CCS |  |  |
| 1.7.9 | Perform Extension Angle Test. - Install the light on the Test Set TD01. Make sure that Test Set is level. - Set the switch “S3” to "ON" position. Wait full extension of the light. - After end of extension set the switch “S3” to the "OFF" position. - Measure of the angle. If the extension angle is not correct, do NOT the adjustment. - Set the switch “S4” to the "ON" position. Wait full retraction of the light. - After end of retraction set the switch “S4” to the "OFF" position. Ref. CMM page 1007, item 1.K. |  |  |  | QM / CCS |  |  |
| 1.7.10 | Perform Post Test Procedures. Certify that all CMJC instructions related to ”TESTING” have been completed in accordance with the CMM. □ All tests passed □ One or more tests NOT passed NOTE: If all tests passed perform Storage Procedure (Ref. CMM page 15001 item 1.A.). If any test fails, mark the Retractable Landing Light as unrepairable. |  |  |  | CCS |  |  |
| 1.8 TEST / CHECK RESULTS |  |  |  |  |  |  |  |
| Item | CMJC | Test / Check | Limits | Value |  | Test Result |  |
| 1 | 1.7.2 | Bonding Test. Ref. CMM page 1002, item 1.D. | Resistance is not more than 10 mΩ. |  |  | □ Pass □ Fail |  |
| 2 | 1.7.3 | Insulation Test. Ref. CMM page 1002, item 1.E. | For P/N’s 4191695 / 4315542 / 4331772 resistance is more than 20 MΩ. |  |  | □ Pass □ Fail |  |
|  |  |  | For P/N 117PE01Y00 resistance is more than 10 MΩ. |  |  |  |  |
| 3 | 1.7.4 | Extension Test. Ref. CMM page 1004, item 1.F. | Light operates an "Extension" mode. | N/R |  | □ Pass □ Fail |  |
|  |  |  | Extension time is less than 10 sec. |  |  | □ Pass □ Fail |  |
|  |  |  | Current consumption is less than 2.4 A. |  |  | □ Pass □ Fail |  |
|  |  |  | After end of extension DMM indicates "0". | N/R |  | □ Pass □ Fail |  |
| 4 | 1.7.5 | Retraction Test. Ref. CMM page 1004, item 1.G. | Light operates a "Retraction" mode. | N/R |  | □ Pass □ Fail |  |
|  |  |  | Retraction time is less than 10 sec. |  |  | □ Pass □ Fail |  |
|  |  |  | Current consumption is less than 1.9 A. |  |  | □ Pass □ Fail |  |
|  |  |  | After end of retraction DMM indicates "0". | N/R |  | □ Pass □ Fail |  |
| 5 | 1.7.6 | Relay Test. Ref. CMM page 1005, item 1.H. | After end of extension relay is energized. | N/R |  | □ Pass □ Fail |  |
|  |  |  | After start of retraction relay is de-energized. | N/R |  | □ Pass □ Fail |  |
| 6 | 1.7.7 | Lamp Test. Ref. CMM pages 1005-1006, item 1.I. | Lamp comes "ON". | N/R |  | □ Pass □ Fail |  |
|  |  |  | For P/N’s 4191695 / 4315542 / 4331772 current consumption is less than 6.5 A. |  |  | □ Pass □ Fail |  |
|  |  |  | For P/N 117PE01Y00 current consumption is less than 2 A. |  |  |  |  |
|  |  |  | After start of retraction lamp goes "OFF". | N/R |  | □ Pass □ Fail |  |
| 7 | 1.7.8 | Indication Switch Test. Ref. CMM page 1006, item 1.J. | Circuit is open (“Lamp” indicator is “OFF”) when light is in retracted position. | N/R |  | □ Pass □ Fail |  |
|  |  |  | The circuit is closed (“Lamp” indicator is “ON”) when light is in extended position or extended / retracted. | N/R |  | □ Pass □ Fail |  |
| 8 | 1.7.9 | Extension Angle Test. Ref. CMM page 1007, item 1.K. | For P/N 4191695 extend angle is 83° to 85°. |  |  | □ Pass □ Fail |  |
|  |  |  | For P/N’s 4315542 / 117PE01Y00 extend angle is 88° to 90°. |  |  |  |  |
|  |  |  | For P/N 4331772 extend angle is 90° to 91°. |  |  |  |  |
| 2. RELEASE RECORDS |  |  |  |  |  |  |  |
| 2.1 | Even shift of technological operations were done during maintenance process, I certify that this is not finally result on component condition at the moment of release to service and not affect its airworthiness. I certify that: - Work performed; - □ Work Package closed □ Work Package NOT closed - A general verification is carried out to ensure that the component is clear of all tools, equipment and any extraneous parts or material; - □ Release Certificate Issued □ Release Certificate NOT Issued |  |  |  | CCS |  |  |

3. WORK DETAILS

| ITEM CHANGE LIST (for Items without s/n) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPL № | P/N |  | Description |  |  | QTY |  | Reason |  |  |  | Detected by Sign/Date |  |
| ITEM CHANGE LIST (for Items with s/n) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IPL № | P/N |  | Description | S/N rem |  | S/N inst |  |  | Reason |  |  | Detected by Sign / Date |  |
| NOTE (completed in case of P/Ns of removed and installed items do not matched) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REPAIR ACTION LIST Filled only if any actions from REPAIR part of CMM was DET- detected, PFMD*-performed and VRF**-verified) * -marked as N/A in case by Contracted MRO performance, * and **- filled by CС/S only |  |  |  |  |  |  |  |  |  |  |  |  |  |
| № | IPL № | Repair Action |  |  | M/H | CMM/SPM Reference |  |  |  | Sign / Date |  |  |  |
|  |  |  |  |  |  |  |  |  |  | DET | PFMD* |  | VRF** |
| ADDITIONAL INSPECTIONS LIST (NDT, battery supplementary tests etc) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| № | Report № |  | Inspection action |  |  | M/H | CMM/SPM Reference |  |  |  |  | Sign / Date |  |
| ADD |  |

| 4. MATERIAL USED (used specification or brand where are several allowable alternatives, should be colored) |  |  |  |
| --- | --- | --- | --- |
| № | Specification | Description | Serviceable Tag No |