- This STATEMENT confirms that involved in a process technical personnel well familiar with the technical documentation used during maintenance process and well recognize that this job card may be executed in a not strict order of step-by-step sequence. This shift of technological operations will not result on component condition at the moment of release to service and will not affect its airworthiness.

- Obey all warnings, cautions & notes listed in CMM/SPM sections that are in use per CMJC items.

- Use tool and equipment (including alternative) IAW “Перечень АТИ ЦТОК”.

| - ACCOMPLISHMENT INSTRUCTIONS 1.1 PREPARATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1.1 | Write down used Measurement Tools. Description; ID Number. Digital multimeter (Fluke 87). Digital inclinometer (RGK U-66). Stopwatch. Ref. Перечень АТИ ЦТОК. | QM / CCS |  |  |
| 1.1.2 | Make sure that the ambient conditions are suitable for testing. Description; Limit; Acceptability. Temperature; 13 to 33 °C; □ Ok □ NOT Ok. Relative humidity; 10 to 90 %. Atmospheric pressure; 610 to 775 mmHg. Ref. CMM page 1002, item 2.A(3). | QM / CCS |  |  |
| 1.1.3 | Perform identify implemented SB’s based on Identification Label and Service Bulletin List. Ref. CMM page SBL-1. |  |  |  | QM / CCS |  |  |
| 1.2 TESTING ATTENTION: Before tests visually inspect the light for signs of missing, broken, bent or cracked parts. If any damaged is found don’t perform tests and mark “N/R” in the Test / Check Results Table all items. ATTENTION: Conduct all tests in the order presented. If any test fails abort the testing and mark “N/R” in the Test / Check Results Table next items. ATTENTION: All test results must be recorded in the Test / Check Results Table. NOTE: The required values / states and their limits are specified in the Test / Check Results Table. |  |  |  |  |  |  |  |
| 1.2.1 | Perform Test Preparation. - Install the light on the Load Fixture IF1610-AT. Make sure that Load Fixture is level (-0,1 to +0,1 degree). - Connect light to the Functional Tester RLTS01-AT. Make sure that the switch “MODE” in the “RETRACT” position and switch “LINE SELECT” in the “112,5” position. Ref. CMM page 1003, item 2.C. |  |  |  | QM / CCS |  |  |
| 1.2.2 | Perform Adjust of the Adjusting Screw. - Adjust the adjustment screw fully counterclockwise. Ref. CMM page 1004, item 2.D(4). |  |  |  | QM / CCS |  |  |
| 1.2.3 | Perform No Load Condition EXTEND Test 1. - Set the switch “MODE” in the “EXTEND” position. - Set the switch “POWER ON” to "ON" position. Wait full extension of the light. - After end of extension set the switch “POWER ON” to the "OFF" position. Ref. CMM page 1005, item 2.D(5-7). |  |  |  | QM / CCS |  |  |
| 1.2.4 | Perform No Load Condition RETRACT Test 1. - Set the switch “MODE” in the “RETRACT” position. - Set the switch “POWER ON” to "ON" position. Wait full retraction of the light. - After end of retraction set the switch “POWER ON” to the "OFF" position. Ref. CMM page 1005, item 2.D(8-10). |  |  |  | QM / CCS |  |  |
| 1.2.5 | Perform Adjust of the Adjusting Screw. - Adjust the adjustment screw fully clockwise. Ref. CMM page 1005, item 2.D(11). |  |  |  | QM / CCS |  |  |
| 1.2.6 | Perform No Load Condition EXTEND Test 2. - Set the switch “MODE” in the “EXTEND” position. - Set the switch “POWER ON” to "ON" position. Wait full extension of the light. - After end of extension set the switch “POWER ON” to the "OFF" position. Ref. CMM page 1005, item 2.D(12-14). |  |  |  | QM / CCS |  |  |
| 1.2.7 | Perform No Load Condition RETRACT Test 2. - Set the switch “MODE” in the “RETRACT” position. - Set the switch “POWER ON” to "ON" position. Wait full retraction of the light. - After end of retraction set the switch “POWER ON” to the "OFF" position. Ref. CMM page 1005, item 2.D(15-17). |  |  |  | QM / CCS |  |  |
| 1.2.8 | Perform Adjust of the Adjusting Screw. - Adjust the adjustment screw counterclockwise to the middle of travel. Ref. CMM page 1005, item 2.D(18). |  |  |  | QM / CCS |  |  |
| 1.2.9 | Perform No Load Condition EXTEND-ON Test 1. - Set the switch “MODE” in the “LIGHT” position. - Set the switch “POWER ON” to "ON" position. Wait full extension of the light. - After end of extension set the switch “POWER ON” to the "OFF" position. Ref. CMM pages 1005-1006, item 2.D(19-21). |  |  |  | QM / CCS |  |  |
| 1.2.10 | Perform No Load Condition RETRACT Test 3. - Set the switch “MODE” in the “RETRACT” position. - Set the switch “POWER ON” to "ON" position. Wait full retraction of the light. - After end of retraction set the switch “POWER ON” to the "OFF" position. Ref. CMM page 1006, item 2.D(22-24). |  |  |  | QM / CCS |  |  |
| 1.2.11 | Perform Load Condition EXTEND Test. - Install 20kg weight on the Load Fixture. - Set the switch “MODE” in the “EXTEND” position. - Set the switch “POWER ON” to "ON" position. Wait full extension of the light. - After end of extension set the switch “POWER ON” to the "OFF" position. Ref. CMM page 1006, item 2.D(25-27). |  |  |  | QM / CCS |  |  |
| 1.2.12 | Perform Load Condition RETRACT Test 1. - Set the switch “MODE” in the “RETRACT” position. - Set the switch “POWER ON” to "ON" position. Wait full retraction of the light. - After end of retraction set the switch “POWER ON” to the "OFF" position. Ref. CMM page 1006, item 2.D(28-30). |  |  |  | QM / CCS |  |  |
| 1.2.13 | Perform Load Condition EXTEND-ON Test. - Set the switch “MODE” in the “LIGHT” position. - Set the switch “POWER ON” to "ON" position. Wait full extension of the light. - Set the switch “LINE SELECT” in the “109” position. - Set the switch “LINE SELECT” in the “105.5” position. - Set the switch “LINE SELECT” in the “112.5” position. Current consumption should be what it was in the second step of this test. - Set the switch “POWER ON” to the "OFF" position. Ref. CMM pages 1006-1007, item 2.D(31-37). |  |  |  | QM / CCS |  |  |
| 1.2.14 | Perform Load Condition RETRACT Test 2. - Set the switch “MODE” in the “RETRACT” position. - Set the switch “POWER ON” to "ON" position. Wait full retraction of the light. - After end of retraction set the switch “POWER ON” to the "OFF" position. Ref. CMM page 1007, item 2.D(38-41). |  |  |  | QM / CCS |  |  |
| 1.2.15 | Perform Post Test Procedures. Certify that all CMJC instructions related to ”TESTING” have been completed in accordance with the CMM. □ All tests passed □ One or more tests NOT passed NOTE: If any test fails go to CMJC 02-334207-XX (if required). |  |  |  | CCS |  |  |
| 1.3 TEST / CHECK RESULTS |  |  |  |  |  |  |  |
| Item | CMJC | Test / Check | Limits | Value |  | Test Result |  |
| 1 | 1.2.3 | No Load Condition EXTEND Test 1. Ref. CMM page 1005, item 2.D(5-7). | Canopy extend with the light OFF. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Motor turn OFF when fully extended. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Extend angle is 76.0° or less. |  |  | □ Pass □ Fail □ N/R |  |
| 2 | 1.2.4 | No Load Condition RETRACT Test 1. Ref. CMM page 1005, item 2.D(8-10). | Canopy retract with the light OFF. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Motor turn OFF when fully retracted. | N/R |  | □ Pass □ Fail □ N/R |  |
| 3 | 1.2.6 | No Load Condition EXTEND Test 2. Ref. CMM page 1005, item 2.D(12-14). | Canopy extend with the light OFF. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Motor turn OFF when fully extended. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Extend angle is 82.0° or more. |  |  | □ Pass □ Fail □ N/R |  |
| 4 | 1.2.7 | No Load Condition RETRACT Test 2. Ref. CMM page 1005, item 2.D(15-17). | Canopy retract with the light OFF. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Motor turn OFF when fully retracted. | N/R |  | □ Pass □ Fail □ N/R |  |
| 5 | 1.2.9 | No Load Condition EXTEND-ON Test 1. Ref. CMM pages 1005-1006, item 2.D(19-21). | Canopy extend with the light ON. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Motor turn OFF when fully extended. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Extend angle is 78.4 to 79.4°. |  |  | □ Pass □ Fail □ N/R |  |
| 6 | 1.2.10 | No Load Condition RETRACT Test 3. Ref. CMM page 1006, item 2.D(22-24). | Canopy retract with the light OFF. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Motor turn OFF when fully retracted. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Retract angle is -1.0 to +1.0°. |  |  | □ Pass □ Fail □ N/R |  |
| 7 | 1.2.11 | Load Condition EXTEND Test. Ref. CMM page 1006, item 2.D(25-27). | Canopy extend with the light OFF. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Motor turn OFF when fully extended. | N/R |  | □ Pass □ Fail □ N/R |  |
| 8 | 1.2.12 | Load Condition RETRACT Test 1. Ref. CMM page 1006, item 2.D(28-30). | Canopy retract with the light OFF. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Motor turn OFF when fully retracted. | N/R |  | □ Pass □ Fail □ N/R |  |
| 9 | 1.2.13 | Load Condition EXTEND-ON Test. Ref. CMM pages 1006-1007, item 2.D(31-37). | Canopy extend with the light ON. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Motor turn OFF when fully extended. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Extension time is 15 sec maximum. |  |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Current on 112.5V is 4.4 to 5.4 A. |  |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Current on 109V is current on 112.5V + 0.15A minimum. |  |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Current on 105V is current on 109V + 0.15A minimum. |  |  | □ Pass □ Fail □ N/R |  |
| 10 | 1.2.14 | Load Condition RETRACT Test 1. Ref. CMM page 1007, item 2.D(38-41). | Canopy retract with the light OFF. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Motor turn OFF when fully retracted. | N/R |  | □ Pass □ Fail □ N/R |  |
|  |  |  | Retraction time is 15 sec maximum. |  |  | □ Pass □ Fail □ N/R |  |
| 2. RELEASE RECORDS |  |  |  |  |  |  |  |
| 2.1 | Even shift of technological operations were done during maintenance process, I certify that this is not finally result on component condition at the moment of release to service and not affect its airworthiness. I certify that: - Work performed; - □ Work Package closed □ Work Package NOT closed - A general verification is carried out to ensure that the component is clear of all tools, equipment and any extraneous parts or material; - □ Release Certificate Issued □ Release Certificate NOT Issued |  |  |  | CCS |  |  |

3. WORK DETAILS

| ITEM CHANGE LIST (for Items without s/n) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPL № | P/N |  | Description |  |  | QTY |  | Reason |  |  |  | Detected by Sign/Date |  |
| ITEM CHANGE LIST (for Items with s/n) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IPL № | P/N |  | Description | S/N rem |  | S/N inst |  |  | Reason |  |  | Detected by Sign / Date |  |
| NOTE (completed in case of P/Ns of removed and installed items do not matched) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REPAIR ACTION LIST Filled only if any actions from REPAIR part of CMM was DET- detected, PFMD*-performed and VRF**-verified) * -marked as N/A in case by Contracted MRO performance, * and **- filled by CС/S only |  |  |  |  |  |  |  |  |  |  |  |  |  |
| № | IPL № | Repair Action |  |  | M/H | CMM/SPM Reference |  |  |  | Sign / Date |  |  |  |
|  |  |  |  |  |  |  |  |  |  | DET | PFMD* |  | VRF** |
| ADDITIONAL INSPECTIONS LIST (NDT, battery supplementary tests etc) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| № | Report № |  | Inspection action |  |  | M/H | CMM/SPM Reference |  |  |  |  | Sign / Date |  |
| ADD |  |

| 4. MATERIAL USED (used specification or brand where are several allowable alternatives, should be colored) |  |  |  |
| --- | --- | --- | --- |
| № | Specification | Description | Serviceable Tag No |