- This STATEMENT confirms that involved in a process technical personnel well familiar with the technical documentation used during maintenance process and well recognize that this job card may be executed in a not strict order of step-by-step sequence. This shift of technological operations will not result on component condition at the moment of release to service and will not affect its airworthiness.

- Obey all warnings, cautions & notes listed in CMM/SPM sections that are in use per CMJC items.

- Use tool and equipment (including alternative) IAW “Перечень АТИ ЦТОК”.

| - ACCOMPLISHMENT INSTRUCTIONS 1.1 PREPARATION |  |  |  |
| --- | --- | --- | --- |
| 1.1.1 | Write down used Measurement Tools. Description; ID Number. Digital multimeter (Fluke 87). Digital inclinometer (RGK U-66). Milliohmmeter (AKIP-6301). Megohmmeter (GPT-79603). Stopwatch. Ref. Перечень АТИ ЦТОК. | QM / CCS |  |
| 1.1.2 | Make sure that the ambient conditions are suitable for testing. Description; Limit; Acceptability. Temperature; 25 ± 10 °C; □ Ok □ NOT Ok. Relative humidity; Less than 85 %. Atmospheric pressure; 84 to107 kPa. Ref. CMM page 1002, item 1.B(3). | QM / CCS |  |
| 1.2 TESTING ATTENTION: All test results must be recorded in the Test / Check Results Table. NOTE: The required values / states and their limits are specified in the Test / Check Results Table. |  |  |  |
| 1.2.1 | Perform Bonding Test. With a Milliohmmeter measure the resistance between the connector pin “J” and the ground studs. Ref. CMM page 1002, item 1.D. | QM / CCS |  |
| 1.2.2 | Perform Insulation Test. For P/N’s 4191695 / 4315542 / 4331772. Connect all the connectors pins together except “J” and apply 500 VDC with a Megohmmeter between all the connected pins and the ground studs. For P/N 117PE01Y00. Connect all the connectors pins (A-B-C-F-H-K-L) together and apply 500 VDC with a Megohmmeter between all the connected pins and the casing of the light. Ref. CMM page 1002, item 1.E. | QM / CCS |  |

| 1.2.3 | Perform Extension Test. - Connect light to the Functional Tester RLTS01-AT. Make sure that the switches “S1”, “S2”, “S3” and “S4” in the “OFF” position. - Set the switch “S3” to "ON" position. Wait full extension of the light. - After end of extension set the switch “S3” to the "OFF" position. Ref. CMM page 1004, item 1.F. |  |  |  | QM / CCS |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1.2.4 | Perform Retraction Test. - Set the switch “S4” to the "ON" position. Wait full retraction of the light. - After end of retraction set the switch “S4” to the "OFF" position. Ref. CMM page 1004, item 1.G. |  |  |  | QM / CCS |  |
| 1.2.5 | Perform Relay Test. - Set the switch “S2” and “S3” to the "ON" position. Wait full extension of the light. - After end of extension set the switch “S3” to the "OFF" position. - Set the switch “S4” to the "ON" position. Wait full retraction of the light. - After end of retraction set the switch “S2” and “S4” to the "OFF" position. Ref. CMM page 1005, item 1.H. |  |  |  | QM / CCS |  |
| 1.2.6 | Perform Lamp Test. - Set the switch “S1”, “S2” and “S3” to the "ON" position. Wait full extension of the light. - After end of extension set the switch “S3” to the "OFF" position. - Set the switch “S4” to the "ON" position. Wait full retraction of the light. - After end of retraction set the switch “S1”, “S2” and “S4” to the "OFF" position. Ref. CMM pages 1005-1006, item 1.I. |  |  |  | QM / CCS |  |
| 1.2.7 | Perform Indication Switch Test. - Set the switch “S3” to "ON" position. Wait full extension of the light. - After end of extension set the switch “S3” to the "OFF" position. - Set the switch “S4” to the "ON" position. Wait full retraction of the light. - After end of retraction set the switch “S4” to the "OFF" position. Ref. CMM page 1006, item 1.J. |  |  |  | QM / CCS |  |
| 1.2.8 | Perform Extension Angle Test. - Install the light on the Test Set TD01. Make sure that Test Set is level. - Set the switch “S3” to "ON" position. Wait full extension of the light. - After end of extension set the switch “S3” to the "OFF" position. - Measure of the angle. If the extension angle is not correct, do NOT the adjustment. - Set the switch “S4” to the "ON" position. Wait full retraction of the light. - After end of retraction set the switch “S4” to the "OFF" position. Ref. CMM page 1007, item 1.K. |  |  |  | QM / CCS |  |
| 1.2.9 | Perform Post Test Procedures. Certify that all CMJC instructions related to ”TESTING” have been completed in accordance with the CMM. □ All tests passed □ One or more tests NOT passed NOTE: If all tests passed perform Storage Procedure (Ref. CMM page 15001 item 1.A.). If any test fails go to CMJC 02-334212-XX (if required). |  |  |  | CCS |  |
| 1.3 TEST / CHECK RESULTS |  |  |  |  |  |  |
| Item | CMJC | Test / Check | Limits | Value | Test Result |  |
| 1 | 1.2.1 | Bonding Test. Ref. CMM page 1002, item 1.D. | Resistance is not more than 10 mΩ. |  | □ Pass □ Fail |  |
| 2 | 1.2.2 | Insulation Test. Ref. CMM page 1002, item 1.E. | For P/N’s 4191695 / 4315542 / 4331772 resistance is more than 20 MΩ. |  | □ Pass □ Fail |  |
|  |  |  | For P/N 117PE01Y00 resistance is more than 10 MΩ. |  |  |  |

| 3 | 1.2.3 | Extension Test. Ref. CMM page 1004, item 1.F. | Light operates an "Extension" mode. | N/R | □ Pass □ Fail |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Extension time is less than 10 sec. |  | □ Pass □ Fail |  |
|  |  |  | Current consumption is less than 2,4 A. |  | □ Pass □ Fail |  |
|  |  |  | After end of extension DMM indicates "0". | N/R | □ Pass □ Fail |  |
| 4 | 1.2.4 | Retraction Test. Ref. CMM page 1004, item 1.G. | Light operates a "Retraction" mode. | N/R | □ Pass □ Fail |  |
|  |  |  | Retraction time is less than 10 sec. |  | □ Pass □ Fail |  |
|  |  |  | Current consumption is less than 1,9 A. |  | □ Pass □ Fail |  |
|  |  |  | After end of retraction DMM indicates "0". | N/R | □ Pass □ Fail |  |
| 5 | 1.2.5 | Relay Test. Ref. CMM page 1005, item 1.H. | After end of extension relay is energized. | N/R | □ Pass □ Fail |  |
|  |  |  | After start of retraction relay is de-energized. | N/R | □ Pass □ Fail |  |
| 6 | 1.2.6 | Lamp Test. Ref. CMM pages 1005-1006, item 1.I. | Lamp comes "ON". | N/R | □ Pass □ Fail |  |
|  |  |  | For P/N’s 4191695 / 4315542 / 4331772 current consumption is less than 6,5 A. |  | □ Pass □ Fail |  |
|  |  |  | For P/N 117PE01Y00 current consumption is less than 2 A. |  |  |  |
|  |  |  | After start of retraction lamp goes "OFF". | N/R | □ Pass □ Fail |  |
| 7 | 1.2.7 | Indication Switch Test. Ref. CMM page 1006, item 1.J. | Circuit is open (“Lamp” indicator is “OFF”) when light is in retracted position. | N/R | □ Pass □ Fail |  |
|  |  |  | The circuit is closed (“Lamp” indicator is “ON”) when light is in extended position or extended / retracted. | N/R | □ Pass □ Fail |  |
| 8 | 1.2.8 | Extension Angle Test. Ref. CMM page 1007, item 1.K. | For P/N 4191695 extend angle is 83° to 85°. |  | □ Pass □ Fail |  |
|  |  |  | For P/N’s 4315542 / 117PE01Y00 extend angle is 88° to 90°. |  |  |  |
|  |  |  | For P/N 4331772 extend angle is 90° to 91°. |  |  |  |
| 2. RELEASE RECORDS |  |  |  |  |  |  |
| 2.1 | Even shift of technological operations were done during maintenance process, I certify that this is not finally result on component condition at the moment of release to service and not affect its airworthiness. I certify that: - Work performed; - □ Work Package closed □ Work Package NOT closed - A general verification is carried out to ensure that the component is clear of all tools, equipment and any extraneous parts or material; - □ Release Certificate Issued □ Release Certificate NOT Issued |  |  |  | CCS |  |

3. WORK DETAILS

| ITEM CHANGE LIST (for Items without s/n) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPL № | P/N |  | Description |  |  | QTY |  | Reason |  |  |  | Detected by Sign/Date |  |
| ITEM CHANGE LIST (for Items with s/n) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IPL № | P/N |  | Description | S/N rem |  | S/N inst |  |  | Reason |  |  | Detected by Sign / Date |  |
| NOTE (completed in case of P/Ns of removed and installed items do not matched) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REPAIR ACTION LIST Filled only if any actions from REPAIR part of CMM was DET- detected, PFMD*-performed and VRF**-verified) * -marked as N/A in case by Contracted MRO performance, * and **- filled by CС/S only |  |  |  |  |  |  |  |  |  |  |  |  |  |
| № | IPL № | Repair Action |  |  | M/H | CMM/SPM Reference |  |  |  | Sign / Date |  |  |  |
|  |  |  |  |  |  |  |  |  |  | DET | PFMD* |  | VRF** |
| ADDITIONAL INSPECTIONS LIST (NDT, battery supplementary tests etc) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| № | Report № |  | Inspection action |  |  | M/H | CMM/SPM Reference |  |  |  |  | Sign / Date |  |
| ADD |  |

| 4. MATERIAL USED (used specification or brand where are several allowable alternatives, should be colored) |  |  |  |
| --- | --- | --- | --- |
| № | Specification | Description | Serviceable Tag No |