- This STATEMENT confirms that involved in a process technical personnel well familiar with the technical documentation used during maintenance process and well recognize that this job card may be executed in a not strict order of step-by-step sequence. This shift of technological operations will not result on component condition at the moment of release to service and will not affect its airworthiness.

- Obey all warnings, cautions & notes listed in CMM/SPM sections that are in use per CMJC items.

- Use tool and equipment (including alternative) IAW “Перечень АТИ ЦТОК”.

| - ACCOMPLISHMENT INSTRUCTIONS 1.1 PREPARATION |  |  |  |
| --- | --- | --- | --- |
| 1.1.1 | Write down used Measurement Tools and Torque Tools. Description; ID Number. Digital multimeter. Torque Range; ID Number. 0,5 - 2,5 Nm. 2 - 10 Nm. Ref. Перечень АТИ ЦТОК. | QM / CCS |  |
| 1.1.2 | Perform identify possible corrective actions based on CMJC 01-334230-XX test results (if applicable) or all available information on the malfunction and Fault Isolation Table. Ref. CMM page 1003, item 2(D). | QM / CCS |  |
| 1.1.3 | Perform identify implemented SB’s based on Identification Label and Service Bulletin List. Ref. CMM page SBL-1. | QM / CCS |  |
| 1.2 DISASSEMBLY |  |  |  |
| 1.2.1 | Perform Disassembly. ATTENTION: Do NOT do more disassembly that are necessary. Ref. CMM pages 3002-3003 item 2.B. | QM / CCS |  |
| 1.3 CLEANING |  |  |  |
| 1.3.1 | Perform Cleaning. - Remove dirt and dust from all external parts with a soft-bristle brush. - Remove oil and grease with aqueous cleaner. - Remove unwanted material on painted surfaces that show index marks, numerals, or letters with a moist cloth. - Remove dirt and dust around the connector pins with air supply with an AIR-IONIZING GUN only. - Remove dirt and dust from all electrical parts with a soft-bristle brush and clean its (except lamp) with aqueous cleaner. - Clean lens and lamp with cleaning cloth. - Remove a thin layer of surface corrosion with correct abrasive cloth (if required). - Clean metal parts with aqueous cleaner and dry it’s with air supply gun. Ref. CMM pages 4002-4003 item 2.B-D. | QM / CCS |  |
| 1.4 INSPECTION/CHECK ATTENTION: All replaced parts must be recorded in the Item Change List. ATTENTION: After replacing the part, repeat the corresponding step of the CMJC. |  |  |  |
| 1.4.1 | Perform Visual Check. - Check for nicks, dings, scratches or small burrs deeper than 0.8 mm. - Check for corrosion deeper than 0.8 mm or more than 20% of surface. - Check painted parts for scratches, peeling, flaking or chipping. - Check threaded parts for distortion or stripping. - Check threaded inserts for stripping and tight fit. - Check electrical parts for cracks, distortion, bent pins or signs of burned connections. - Check electrical wires for cracks, breaks or deterioration of insulation. - Check lens for cracks and discoloration. - Check transformer assembly for evidence of overheating. - Check housing for dents, cracks or breaks. Ref. CMM pages 5002-5003 items 2.B. | QM / CCS |  |
| 1.4.2 | Certify that all CMJC instructions related to ”INSPECTION/CHECK” have been completed in accordance with the CMM. | CCS |  |
| 1.5 REPAIR ATTENTION: All used consumable materials and spare parts must be recorded in the Material Used Table and Item Change List. |  |  |  |
| 1.5.1 | Perform Surface Repair. □ Performed □ N/R Ref. CMM page 6004 item 2.D. | QM / CCS |  |
| 1.5.2 | Perform Lens Assembly Repair. □ Performed □ N/R Ref. CMM pages 6004-6005 item 2.E. | QM / CCS |  |
| 1.5.3 | Perform Housing Assembly Repair. □ Performed □ N/R Ref. CMM page 6005 item 2.F. | QM / CCS |  |
| 1.5.4 | Perform Support Assembly Repair. □ Performed □ N/R Ref. CMM page 6005 item 2.G. | QM / CCS |  |
| 1.5.5 | Perform Electrical Wiring Repair. □ Performed □ N/R Ref. CMM page 6006 item 2.H. | QM / CCS |  |
| 1.5.6 | Perform Housing Subassembly Repair. □ Performed □ N/R Ref. CMM pages 6006-6007 item 2.I. | QM / CCS |  |
| 1.5.7 | Certify that all CMJC instructions related to ”REPAIR” have been completed in accordance with the CMM. | CCS |  |
| 1.6 ASSEMBLY |  |  |  |
| 1.6.1 | Perform Assembly. NOTE: For required torque values see “MECHANICAL REPAIR” section of the “SOPM 20-00-03 Standard Repair Procedures for Honeywell Avionics Equipment Instruction Manual”. Ref. CMM pages 7002-7004 items 2.B(1-13), SOPM page 5-73 item 2.P. | QM / CCS |  |
| 1.6.2 | Certify that all CMJC instructions related to ”ASSEMBLY” have been completed in accordance with the CMM. | CCS |  |
| 1.7 TESTING ATTENTION: All test results must be recorded in the Test / Check Results Table. NOTE: The required values / states and their limits are specified in the Test / Check Results Table. |  |  |  |

| 1.7.1 | Make sure that the ambient conditions are suitable for testing. Description; Limit; Acceptability. Temperature; 18 to 29 °C; □ Ok □ NOT Ok. Ref. CMM page 1002, item 2.A(2). | QM / CCS |  |  |
| --- | --- | --- | --- | --- |
| 1.7.2 | Perform Operational Test. - Connect light to the Universal Aircraft Lights Activation Box P/N AT0003.0. - With the "Component" rotary selector select "334230". - Use appropriate touch button for connect and disconnect the circuit. Ref. CMM page 1002, item 2.B. |  |  |  | QM / CCS |  |  |
| 1.7.3 | Perform Post Test Procedures. Certify that all CMJC instructions related to ”TESTING” have been completed in accordance with the CMM. □ All tests passed □ One or more tests NOT passed Note: If any test fails, mark the Electric Tail Logo Floodlight as unrepairable. |  |  |  | CCS |  |  |
| 1.8 TEST / CHECK RESULTS |  |  |  |  |  |  |  |
| Item | CMJC | Test / Check | Limits | Value |  | Test Result |  |
| 1 | 1.7.2 | Operational Test. Ref. CMM page 1002, item 2.B. | Lamp comes "ON" with no variance in intensity. | N/R |  | □ Pass □ Fail |  |
| 2. RELEASE RECORDS |  |  |  |  |  |  |  |
| 2.1. | Even shift of technological operations were done during maintenance process, I certify that this is not finally result on component condition at the moment of release to service and not affect its airworthiness. I certify that: - Work performed; - □ Work Package closed □ Work Package NOT closed - A general verification is carried out to ensure that the component is clear of all tools, equipment and any extraneous parts or material; - □ Release Certificate Issued □ Release Certificate NOT Issued |  |  |  | CCS |  |  |

3. WORK DETAILS

| ITEM CHANGE LIST (for Items without s/n) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPL № | P/N |  | Description |  |  | QTY |  | Reason |  |  |  | Detected by Sign/Date |  |
| ITEM CHANGE LIST (for Items with s/n) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IPL № | P/N |  | Description | S/N rem |  | S/N inst |  |  | Reason |  |  | Detected by Sign / Date |  |
| NOTE (completed in case of P/Ns of removed and installed items do not matched) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REPAIR ACTION LIST Filled only if any actions from REPAIR part of CMM was DET- detected, PFMD*-performed and VRF**-verified) * -marked as N/A in case by Contracted MRO performance, * and **- filled by CС/S only |  |  |  |  |  |  |  |  |  |  |  |  |  |
| № | IPL № | Repair Action |  |  | M/H | CMM/SPM Reference |  |  |  | Sign / Date |  |  |  |
|  |  |  |  |  |  |  |  |  |  | DET | PFMD* |  | VRF** |
| ADDITIONAL INSPECTIONS LIST (NDT, battery supplementary tests etc) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| № | Report № |  | Inspection action |  |  | M/H | CMM/SPM Reference |  |  |  |  | Sign / Date |  |
| ADD |  |

| 4. MATERIAL USED (used specification or brand where are several allowable alternatives, should be colored) |  |  |  |
| --- | --- | --- | --- |
| № | Specification | Description | Serviceable Tag No |