- This STATEMENT confirms that involved in a process technical personnel well familiar with the technical documentation used during maintenance process and well recognize that this job card may be executed in a not strict order of step-by-step sequence. This shift of technological operations will not result on component condition at the moment of release to service and will not affect its airworthiness.

- Obey all warnings, cautions & notes listed in CMM/SPM sections that are in use per CMJC items.

- Use tool and equipment (including alternative) IAW “Перечень АТИ ЦТОК”.

| - ACCOMPLISHMENT INSTRUCTIONS 1.1 PREPARATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1.1 | Write down used Measurement Tools and Torque Tools. Description; ID Number. Digital multimeter (Fluke 87). Milliohmmeter (AKIP-6301). Megohmmeter (GPT-79603). Digital caliper (SCC-I-250-0.01). Digital scales (PVM-3/30-P). Torque Range; ID Number. 0.5 - 2.5 Nm (A.402). Ref. Перечень АТИ ЦТОК. | QM / CCS |  |  |
| 1.1.2 | Perform identify possible corrective actions based on CMJC 01-334303-XX test results (if applicable) or all available information on the malfunction and Fault Isolation Table. Ref. CMM page 1005, item 2. |  |  |  | QM / CCS |  |  |
| 1.1.3 | Perform identify implemented SB’s based on Identification Label and Service Bulletin List. Ref. CMM page SBL Page 1. |  |  |  | QM / CCS |  |  |
| 1.2 DISASSEMBLY |  |  |  |  |  |  |  |
| 1.2.1 | Perform Disassembly. ATTENTION: Do NOT do more disassembly that are necessary. Ref. CMM pages 3002-3004 item 1.D. |  |  |  | QM / CCS |  |  |
| 1.3 CLEANING |  |  |  |  |  |  |  |
| 1.3.1 | Perform Cleaning. - Clean all parts (except electrical parts) with a lightly moist cloth with solvent. - Clean the instruction plate, the amendment plate, the bar code identification label and connector pins with a clean and soft lint-free cloth. - Remove all bonding agent material from the optic support (if necessary). - Clean the autotransformer and the receptacle connector with low-pressure compressed air. - Clean all the threads in which screws were locked using a cloth lightly moist with solvent and dry with low-pressure compressed air. - Clean all the lamp terminals fully and remove the corrosion from the terminal lugs with emery cloth. - Clean the attaching parts with a white spirit solution and dry them with a low-pressure compressed air. Ref. CMM pages 4002-4003 item 1.D. |  |  |  | QM / CCS |  |  |
| 1.4 INSPECTION/CHECK ATTENTION: All replaced parts must be recorded in the Item Change List. ATTENTION: After replacing the part, repeat the corresponding step of the CMJC. |  |  |  |  |  |  |  |
| 1.4.1 | Perform Visual Check. - Check condition of the all parts and the good condition of the anticorrosion protection layers. For cadmium dichromate plating make sure that the layer is constant, fine grained and does not show cracking. - Check the general condition of the assembled parts for damage other than usual wear. - Check the condition of the threads and tapped holes, terminal studs (deformation, oxidation…), optic support (make sure that the protection of the two holes is in a good condition), equipped clamp, lamp, autotransformer, receptacle connector and the gasket. - Check the lamp glass or the deflector glass for scratches. - Check the reflector of SHL for no unsticking (if required). Ref. CMM pages 5001-5002 item 1.D(1,2(a,b)). |  |  |  | QM / CCS |  |  |
| 1.4.2 | Perform Autotransformer Check. - Supply the autotransformer with 115 VAC ± 0.5 V / 400 Hz. - Make sure that the voltmeter reading is equal to 28 ± 0.5 V / 400 Hz. - Make sure that the ammeter reading is 2 A maximum. - Make sure that the autotransformer NOT overheating. Ref. CMM page 5002 item 1.D(2(c)). |  |  |  | QM / CCS |  |  |
| 1.4.3 | Perform Dimensional Check. - Make sure that diameter of the 2 fixation holes is 1/4 - 28 UNF 3B. - Make sure that distance between fixation holes is 21 ± 0.2 mm. Ref. CMM page 5002 item 1.D(3). |  |  |  | QM / CCS |  |  |
| 1.4.4 | Perform Weight Check. - Make sure that weight of light assy is 0,8 kg maximum. Ref. CMM page 5002 item 1.D(4). |  |  |  | QM / CCS |  |  |
| 1.4.5 | Certify that all CMJC instructions related to ”INSPECTION/CHECK” have been completed in accordance with the CMM. |  |  |  | CCS |  |  |
| 1.5 REPAIR ATTENTION: All used consumable materials and spare parts must be recorded in the Material Used Table and Item Change List. |  |  |  |  |  |  |  |
| 1.5.1 | Perform Repair of the Receptacle Connector. □ Performed □ N/R Ref. CMM page 6003 item 1.D(3). |  |  |  | QM / CCS |  |  |
| 1.5.2 | Certify that all CMJC instructions related to ”REPAIR” have been completed in accordance with the CMM. |  |  |  | CCS |  |  |
| 1.6 ASSEMBLY |  |  |  |  |  |  |  |
| 1.6.1 | Perform Assembly. NOTE: For required torque values see “FITS AND CLEARANCES” section. Ref. CMM pages 7002-7008 item 1.C, page 8001 item 1.A. |  |  |  | QM / CCS |  |  |
| 1.6.2 | Certify that all CMJC instructions related to ”ASSEMBLY” have been completed in accordance with the CMM. |  |  |  | CCS |  |  |
| 1.7 TESTING ATTENTION: All test results must be recorded in the Test / Check Results Table. NOTE: The required values / states and their limits are specified in the Test / Check Results Table. |  |  |  |  |  |  |  |
| 1.7.1 | Make sure that the ambient conditions are suitable for testing. Description; Limit; Acceptability. Temperature; 20 ± 5 °C; □ Ok □ NOT Ok. Relative humidity; Less than 85 %. Atmospheric pressure; 900 to 1100 mbar. Ref. CMM page 1002, items 1.C(3). | QM / CCS |  |  |
| 1.7.2 | Perform Operational Test. - Connect of the light to the 115 V/400 Hz power supply. - Disconnect and re-connect the circuit. - Measure the current consumption. Ref. CMM page 1003, item 1.D. |  |  |  | QM / CCS |  |  |
| 1.7.3 | Perform Insulation Test. Connect connectors pins together and apply 500 VDC with a Megohmmeter between all the connected pins and the ground for a minimum 5 seconds. Ref. CMM page 1003, item 1.E. |  |  |  | QM / CCS |  |  |
| 1.7.4 | Perform Bonding Test. With a Milliohmmeter measure the resistance between a position on the optic support and the ground terminal. Ref. CMM page 1004, item 1.F. |  |  |  | QM / CCS |  |  |
| 1.7.5 | Perform Substitute Halogen Lamp Test. - Remove the lens. - Apply a voltage of 28 ± 0.5 VDC on the terminals of the lamp. - Measure the current consumption. - Install the lens. □ Performed □ N/R NOTE: For SHL P/N 4394909 only. Ref. CMM page 1004, item 1.G, page 3002 item 1.D(1(b(1,2))), page 7005-7006 item 1.D(4(b(12,13))). |  |  |  | QM / CCS |  |  |
| 1.7.6 | Perform Post Test Procedures. Certify that all CMJC instructions related to ”TESTING” have been completed in accordance with the CMM. □ All tests passed □ One or more tests NOT passed NOTE: If any test fails, mark the Runway Turn-Off Light as unrepairable. |  |  |  | CCS |  |  |
| 1.8 TEST / CHECK RESULTS |  |  |  |  |  |  |  |
| Item | CMJC | Test / Check | Limits | Value |  | Test Result |  |
| 1 | 1.7.2 | Operational Test. Ref. CMM page 1003, item 1.D. | Lamp comes "ON". | N/R |  | □ Pass □ Fail |  |
|  |  |  | Current consumption is 2 A maximum. |  |  | □ Pass □ Fail |  |
| 2 | 1.7.3 | Insulation Test. Ref. CMM page 1003, item 1.E. | Resistance is more than 100 MΩ. |  |  | □ Pass □ Fail |  |
| 3 | 1.7.4 | Bonding Test. Ref. CMM page 1004, item 1.F. | Resistance is less than 20 mΩ. |  |  | □ Pass □ Fail |  |
| 4 | 1.7.5 | Substitute Halogen Lamp Test. NOTE: For SHL P/N 4394909 only. Ref. CMM page 1004, item 1.G. | Current consumption is 6 A maximum. |  |  | □ Pass □ Fail □ N/A |  |
| 2. RELEASE RECORDS |  |  |  |  |  |  |  |
| 2.1 | Even shift of technological operations were done during maintenance process, I certify that this is not finally result on component condition at the moment of release to service and not affect its airworthiness. I certify that: - Work performed; - □ Work Package closed □ Work Package NOT closed - A general verification is carried out to ensure that the component is clear of all tools, equipment and any extraneous parts or material; - □ Release Certificate Issued □ Release Certificate NOT Issued |  |  |  | CCS |  |  |

3. WORK DETAILS

| ITEM CHANGE LIST (for Items without s/n) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPL № | P/N |  | Description |  |  | QTY |  | Reason |  |  |  | Detected by Sign/Date |  |
| ITEM CHANGE LIST (for Items with s/n) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IPL № | P/N |  | Description | S/N rem |  | S/N inst |  |  | Reason |  |  | Detected by Sign / Date |  |
| NOTE (completed in case of P/Ns of removed and installed items do not matched) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REPAIR ACTION LIST Filled only if any actions from REPAIR part of CMM was DET- detected, PFMD*-performed and VRF**-verified) * -marked as N/A in case by Contracted MRO performance, * and **- filled by CС/S only |  |  |  |  |  |  |  |  |  |  |  |  |  |
| № | IPL № | Repair Action |  |  | M/H | CMM/SPM Reference |  |  |  | Sign / Date |  |  |  |
|  |  |  |  |  |  |  |  |  |  | DET | PFMD* |  | VRF** |
| ADDITIONAL INSPECTIONS LIST (NDT, battery supplementary tests etc) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| № | Report № |  | Inspection action |  |  | M/H | CMM/SPM Reference |  |  |  |  | Sign / Date |  |
| ADD |  |

| 4. MATERIAL USED (used specification or brand where are several allowable alternatives, should be colored) |  |  |  |
| --- | --- | --- | --- |
| № | Specification | Description | Serviceable Tag No |