- This STATEMENT confirms that involved in a process technical personnel well familiar with the technical documentation used during maintenance process and well recognize that this job card may be executed in a not strict order of step-by-step sequence. This shift of technological operations will not result on component condition at the moment of release to service and will not affect its airworthiness.

- Obey all warnings, cautions & notes listed in CMM/SPM sections that are in use per CMJC items.

- Use tool and equipment (including alternative) IAW “Перечень АТИ ЦТОК”.

| - ACCOMPLISHMENT INSTRUCTIONS 1.1 PREPARATION |  |  |  |
| --- | --- | --- | --- |
| 1.1.1 | Write down used Measurement Tools. Description; ID Number. Digital multimeter (Fluke 87). Milliohmmeter (AKIP-6301). Megohmmeter (GPT-79603). Ref. Перечень АТИ ЦТОК. | QM / CCS |  |
| 1.1.2 | Make sure that the ambient conditions are suitable for testing. Description; Limit; Acceptability. Temperature; 25 ± 5 °C; □ Ok □ NOT Ok. Relative humidity; Less than 85 %. Atmospheric pressure; 900 to 1100 mbar. Ref. CMM pages 1002, item 1.C(3). | QM / CCS |  |
| 1.2 TESTING ATTENTION: All test results must be recorded in the Test / Check Results Table. NOTE: The required values / states and their limits are specified in the Test / Check Results Table. |  |  |  |
| 1.2.1 | Perform Operational Test of Take-Off Light (600 W filament). - Connect light to the Activation Box LLAB01-AT. Make sure that the left switch ”Light Type” in the “Take-Off / Taxi” position, right switch ”Light Type” in the “Take-Off” position and switch “ON” in the “OFF” position. - Use “ON” switch for connect and disconnect the circuit. Ref. CMM page 1002, item 1.E. | QM / CCS |  |
| 1.2.2 | Perform Operational Test of Taxi Light (400 W filament). - Set right switch ”Light Type” in the “Taxi” position. - Use “ON” switch for connect and disconnect the circuit. Ref. CMM page 1002, item 1.F. | QM / CCS |  |
| 1.2.3 | Perform Insulation Test. Connect a Megohmmeter between the pins “A”, “B” and “C” connected together of the receptacle connector and a position on the lamp housing for a minimum of 5 seconds. Ref. CMM pages 1003-1004, item 1.G. | QM / CCS |  |
| 1.2.4 | Perform Bonding Test. With a Milliohmmeter measure the resistance between a position on the lamp housing and the ground terminal. Ref. CMM page 1004, item 1.H. | QM / CCS |  |

| 1.2.5 | Perform Post Test Procedures. Certify that all CMJC instructions related to ”TESTING” have been completed in accordance with the CMM. □ All tests passed □ One or more tests NOT passed Note: If any test fails go to CMJC 02-334612-XX (if required). |  |  |  | CCS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.3 TEST / CHECK RESULTS |  |  |  |  |  |  |  |
| Item | CMJC | Test / Check | Limits | Value |  | Test Result |  |
| 1 | 1.2.1 | Operational Test of Take-Off Light. Ref. CMM page 1002, item 1.E. | Lamp comes "ON". | N/R |  | □ Pass □ Fail |  |
|  |  |  | Current consumption is 6.5 A maximum. |  |  | □ Pass □ Fail |  |
| 2 | 1.2.2 | Operational Test of Taxi Light. Ref. CMM page 1002, item 1.F. | Lamp comes "ON". | N/R |  | □ Pass □ Fail |  |
|  |  |  | Current consumption is 4.6 A maximum. |  |  | □ Pass □ Fail |  |
| 3 | 1.2.3 | Insulation Test. Ref. CMM pages 1003-1004, item 1.G. | Resistance is equal to or more than 20 MΩ. |  |  | □ Pass □ Fail |  |
| 4 | 1.2.4 | Bonding Test. Ref. CMM page 1004, item 1.H. | Resistance is not more than 20 mΩ. |  |  | □ Pass □ Fail |  |
| 2. RELEASE RECORDS |  |  |  |  |  |  |  |
| 2.1 | Even shift of technological operations were done during maintenance process, I certify that this is not finally result on component condition at the moment of release to service and not affect its airworthiness. I certify that: - Work performed; - □ Work Package closed □ Work Package NOT closed - A general verification is carried out to ensure that the component is clear of all tools, equipment and any extraneous parts or material; - □ Release Certificate Issued □ Release Certificate NOT Issued |  |  |  | CCS |  |  |

3. WORK DETAILS

| ITEM CHANGE LIST (for Items without s/n) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPL № | P/N |  | Description |  |  | QTY | Reason |  |  |  | Detected by Sign/Date |  |
| ITEM CHANGE LIST (for Items with s/n) |  |  |  |  |  |  |  |  |  |  |  |  |
| IPL № | P/N |  | Description | S/N rem |  | S/N inst |  | Reason |  |  | Detected by Sign / Date |  |
| NOTE (completed in case of P/Ns of removed and installed items do not matched) |  |  |  |  |  |  |  |  |  |  |  |  |
| REPAIR ACTION LIST Filled only if any actions from REPAIR part of CMM was DET- detected, PFMD*-performed and VRF**-verified) * -marked as N/A in case by Contracted MRO performance, * and **- filled by CС/S only |  |  |  |  |  |  |  |  |  |  |  |  |
| № | IPL № | Repair Action |  |  | M/H | CMM/SPM Reference |  |  | Sign / Date |  |  |  |
|  |  |  |  |  |  |  |  |  | DET | PFMD* |  | VRF** |

| ADDITIONAL INSPECTIONS LIST (NDT, battery supplementary tests etc) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| № | Report № | Inspection action |  | M/H | CMM/SPM Reference | Sign / Date |
| ADD |  |

| 4. MATERIAL USED (used specification or brand where are several allowable alternatives, should be colored) |  |  |  |
| --- | --- | --- | --- |
| № | Specification | Description | Serviceable Tag No |