- This STATEMENT confirms that involved in a process technical personnel well familiar with the technical documentation used during maintenance process and well recognize that this job card may be executed in a not strict order of step-by-step sequence. This shift of technological operations will not result on component condition at the moment of release to service and will not affect its airworthiness.

- Obey all warnings, cautions & notes listed in CMM/SPM sections that are in use per CMJC items.

- Use tool and equipment (including alternative) IAW “Перечень АТИ ЦТОК”.

| - ACCOMPLISHMENT INSTRUCTIONS 1.1 PREPARATION |  |  |  |
| --- | --- | --- | --- |
| 1.1.1 | Write down used Measurement Tools and Torque Tools. Description; ID Number. Digital multimeter (Fluke 87). Milliohmmeter (AKIP-6301). Megohmmeter (GPT-79603). Digital caliper (SCC-I-250-0.01). Torque Range; ID Number. 0.5 - 2.5 Nm (A.402). Ref. Перечень АТИ ЦТОК. | QM / CCS |  |
| 1.1.2 | Perform identify possible corrective actions based on CMJC 01-334612-XX test results (if applicable) or all available information on the malfunction and Fault Isolation Table. Ref. CMM page 1005, item 2. | QM / CCS |  |
| 1.2 DISASSEMBLY |  |  |  |
| 1.2.1 | Perform Disassembly. ATTENTION: Do NOT do more disassembly that are necessary. Ref. CMM pages 3001-3002 items 1.D. | QM / CCS |  |
| 1.3 CLEANING |  |  |  |
| 1.3.1 | Perform Cleaning. - Clean all external surfaces (except of electrical parts) using a cloth lightly moist with solvent. - Clean the instruction label, the wiring diagram plate and the name plate using a soft lint-free cloth. - Loosen dust and unwanted material from the connector pins using a low-pressure air jet. - Remove all bonding agent material from the lamp housing (if required). - Clean the autotransformers and the electrical receptacle connector using a low-pressure air jet. - Clean all the threads in which screws were locked using a cloth lightly moist with solvent and dry them with a low-pressure air jet. - Clean all the lamp terminals fully and remove the corrosion from the terminal lugs with emery cloth. - Clean the attaching parts with put them into a white spirit and dry with a low-pressure air jet. Ref. CMM pages 4002-4003 item 1.D. | QM / CCS |  |

| 1.4 INSPECTION/CHECK ATTENTION: All replaced parts must be recorded in the Item Change List. ATTENTION: After replacing the part, repeat the corresponding step of the CMJC. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.4.1 | Perform Visual Check. - Check all parts for the generally condition and good condition of the anticorrosion protection layers (for cadmium dichromated plating, make sure that the layer is constant, fine grained and does not show cracking). - Check the general condition of the assembled parts, to find the possible causes of all damage other than usual wear. - Check for good condition of the threads and tapped holes, lamp housing, clamp, incandescent lamp, autotransformers, receptacle connector and gasket. Ref. CMM pages 5001-5002 item 1.D(1). |  |  |  | QM / CCS |  |  |
| 1.4.2 | Perform Electrical Components Check. - Check for scratches the incandescent lamp glass and the deflector glass. - Connect light to the Activation Box LLAB01-AT. Make sure that the left switch ”Light Type” in the “Take-Off / Taxi” position, right switch ”Light Type” in the “Take-Off” position and switch “ON” in the “OFF” position. - Use “ON” switch for connect and disconnect the circuit. Make sure than the current consumption is 6.5 A maximum and autotransformer (1-70) NOT overheat. - Set right switch ”Light Type” in the “Taxi” position. - Use “ON” switch for connect and disconnect the circuit. Make sure than the current consumption is 4.6 A maximum and autotransformer (1-150) NOT overheat. Ref. CMM page 5002 item 1.D(2). |  |  |  | QM / CCS |  |  |
| 1.4.3 | Perform Dimensional Check. □ Performed □ N/R NOTE: Perform this check only if incorrect dimensions are suspected. Ref. CMM page 5002 item 1.D(3). |  |  |  | QM / CCS |  |  |
| 1.4.4 | Certify that all CMJC instructions related to ”INSPECTION/CHECK” have been completed in accordance with the CMM. |  |  |  | CCS |  |  |
| 1.5 REPAIR ATTENTION: All used consumable materials and spare parts must be recorded in the Material Used Table and Item Change List. |  |  |  |  |  |  |  |
| 1.5.1 | Perform Terminal Lugs Replacement. □ Performed □ N/R Ref. CMM page 6002 item 1.D(1). |  |  |  | QM / CCS |  |  |
| 1.5.2 | Perform Pins Replacement. □ Performed □ N/R Ref. CMM page 6002 item 1.D(2). |  |  |  | QM / CCS |  |  |
| 1.5.3 | Perform Plates and Label Replacement. □ Performed □ N/R Ref. CMM page 6003 item 1.D(2). |  |  |  | QM / CCS |  |  |
| 1.5.4 | Certify that all CMJC instructions related to ”REPAIR” have been completed in accordance with the CMM. |  |  |  | CCS |  |  |
| 1.6 ASSEMBLY |  |  |  |  |  |  |  |
| 1.6.1 | Perform Assembly. NOTE: For required torque values see “FITS AND CLEARANCES” section. Ref. CMM pages 7002-7005 item 1.D, page 8001 item 1.A. |  |  |  | QM / CCS |  |  |
| 1.6.2 | Certify that all CMJC instructions related to ”ASSEMBLY” have been completed in accordance with the CMM. |  |  |  | CCS |  |  |
| 1.7 TESTING ATTENTION: All test results must be recorded in the Test / Check Results Table. NOTE: The required values / states and their limits are specified in the Test / Check Results Table. |  |  |  |  |  |  |  |
| 1.7.1 | Make sure that the ambient conditions are suitable for testing. Description; Limit; Acceptability. Temperature; 25 ± 5 °C; □ Ok □ NOT Ok. Relative humidity; Less than 85 %. Atmospheric pressure; 900 to 1100 mbar. Ref. CMM pages 1002, item 1.C(3). | QM / CCS |  |  |
| 1.7.2 | Perform Operational Test of Take-Off Light (600 W filament). - Connect light to the Activation Box LLAB01-AT. Make sure that the left switch ”Light Type” in the “Take-Off / Taxi” position, right switch ”Light Type” in the “Take-Off” position and switch “ON” in the “OFF” position. - Use “ON” switch for connect and disconnect the circuit. Ref. CMM page 1002, item 1.E. |  |  |  | QM / CCS |  |  |
| 1.7.3 | Perform Operational Test of Taxi Light (400 W filament). - Set right switch ”Light Type” in the “Taxi” position. - Use “ON” switch for connect and disconnect the circuit. Ref. CMM page 1002, item 1.F. |  |  |  | QM / CCS |  |  |
| 1.7.4 | Perform Insulation Test. Connect a Megohmmeter between the pins “A”, “B” and “C” connected together of the receptacle connector and a position on the lamp housing for a minimum of 5 seconds. Ref. CMM pages 1003-1004, item 1.G. |  |  |  | QM / CCS |  |  |
| 1.7.5 | Perform Bonding Test. With a Milliohmmeter measure the resistance between a position on the lamp housing and the ground terminal. Ref. CMM page 1004, item 1.H. |  |  |  | QM / CCS |  |  |
| 1.7.6 | Perform Post Test Procedures. Certify that all CMJC instructions related to ”TESTING” have been completed in accordance with the CMM. □ All tests passed □ One or more tests NOT passed Note: If any test fails, mark the Take-Off and Taxi Light as unrepairable. |  |  |  | CCS |  |  |
| 1.8 TEST / CHECK RESULTS |  |  |  |  |  |  |  |
| Item | CMJC | Test / Check | Limits | Value |  | Test Result |  |
| 1 | 1.7.2 | Operational Test of Take-Off Light. Ref. CMM page 1002, item 1.E. | Lamp comes "ON". | N/R |  | □ Pass □ Fail |  |
|  |  |  | Current consumption is 6.5 A maximum. |  |  | □ Pass □ Fail |  |
| 2 | 1.7.3 | Operational Test of Taxi Light. Ref. CMM page 1002, item 1.F. | Lamp comes "ON". | N/R |  | □ Pass □ Fail |  |
|  |  |  | Current consumption is 4.6 A maximum. |  |  | □ Pass □ Fail |  |
| 3 | 1.7.4 | Insulation Test. Ref. CMM pages 1003-1004, item 1.G. | Resistance is equal to or more than 20 MΩ. |  |  | □ Pass □ Fail |  |
| 4 | 1.7.5 | Bonding Test. Ref. CMM page 1004, item 1.H. | Resistance is not more than 20 mΩ. |  |  | □ Pass □ Fail |  |
| 2. RELEASE RECORDS |  |  |  |  |  |  |  |
| 2.1 | Even shift of technological operations were done during maintenance process, I certify that this is not finally result on component condition at the moment of release to service and not affect its airworthiness. I certify that: - Work performed; - □ Work Package closed □ Work Package NOT closed - A general verification is carried out to ensure that the component is clear of all tools, equipment and any extraneous parts or material; - □ Release Certificate Issued □ Release Certificate NOT Issued |  |  |  | CCS |  |  |

3. WORK DETAILS

| ITEM CHANGE LIST (for Items without s/n) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPL № | P/N |  | Description |  |  | QTY |  | Reason |  |  |  | Detected by Sign/Date |  |
| ITEM CHANGE LIST (for Items with s/n) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IPL № | P/N |  | Description | S/N rem |  | S/N inst |  |  | Reason |  |  | Detected by Sign / Date |  |
| NOTE (completed in case of P/Ns of removed and installed items do not matched) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REPAIR ACTION LIST Filled only if any actions from REPAIR part of CMM was DET- detected, PFMD*-performed and VRF**-verified) * -marked as N/A in case by Contracted MRO performance, * and **- filled by CС/S only |  |  |  |  |  |  |  |  |  |  |  |  |  |
| № | IPL № | Repair Action |  |  | M/H | CMM/SPM Reference |  |  |  | Sign / Date |  |  |  |
|  |  |  |  |  |  |  |  |  |  | DET | PFMD* |  | VRF** |
| ADDITIONAL INSPECTIONS LIST (NDT, battery supplementary tests etc) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| № | Report № |  | Inspection action |  |  | M/H | CMM/SPM Reference |  |  |  |  | Sign / Date |  |
| ADD |  |

| 4. MATERIAL USED (used specification or brand where are several allowable alternatives, should be colored) |  |  |  |
| --- | --- | --- | --- |
| № | Specification | Description | Serviceable Tag No |