- This STATEMENT confirms that involved in a process technical personnel well familiar with the technical documentation used during maintenance process and well recognize that this job card may be executed in a not strict order of step-by-step sequence. This shift of technological operations will not result on component condition at the moment of release to service and will not affect its airworthiness.

- Obey all warnings, cautions & notes listed in CMM/SPM sections that are in use per CMJC items.

- Use tool and equipment (including alternative) IAW “Перечень АТИ ЦТОК”.

| - ACCOMPLISHMENT INSTRUCTIONS 1.1 PREPARATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1.1 | Write down used Measurement Tools. Description; ID Number. Digital multimeter. Milliohmmeter. Megohmmeter. Ref. Перечень АТИ ЦТОК. | QM / CCS |  |  |
| 1.1.2 | Perform identify possible corrective actions based on CMJC 01-334711-XX test results (if applicable) or all available information on the malfunction and Fault Isolation Table. Ref. CMM page 1004, item 2. |  |  |  | QM / CCS |  |  |
| 1.1.3 | Perform identify implemented SB’s based on Identification Label and Service Bulletin List. Ref. CMM page SBL 1/2. |  |  |  | QM / CCS |  |  |
| 1.2 DISASSEMBLY |  |  |  |  |  |  |  |
| 1.2.1 | Perform Disassembly. ATTENTION: Do NOT do more disassembly that are necessary. NOTE: In case of incandescent lamp or autotransformer removal seal P/N 8000115Y00 (item 1A-65) must be discarded and installed new on. Ref. CMM pages 3001-3004 item 1.C. |  |  |  | QM / CCS |  |  |
| 1.3 CLEANING |  |  |  |  |  |  |  |
| 1.3.1 | Perform Cleaning. - Clean all parts (except electrical parts) with a moist cloth. - Clean the autotransformer assy, the receptacle connector and the plates with low-pressure compressed air only. - Clean the lamp terminals fully and remove corrosion from the lugs with emery cloth. - Remove all bonding agent material from housing assy (if necessary). Ref. CMM page 4002 item 1.D. |  |  |  | QM / CCS |  |  |
| 1.4 INSPECTION/CHECK ATTENTION: All replaced parts must be recorded in the Item Change List. ATTENTION: After replacing the part, repeat the corresponding step of the CMJC. |  |  |  |  |  |  |  |
| 1.4.1 | Perform Visual Check. - Check condition of the all parts and the good condition of the anticorrosion protection layers. - Check condition of the optical glass of the light and the deflector glass for scratches. - Check condition of the reflector layer on inner side of the light for unstick area. - Check condition of the turn lock studs for correct operate, without seizure and for no corrosion or significant wear. - Check condition of the retaining ring for wear and for a tight fit. - Check condition of the all terminal lugs for correctly crimping. Ref. CMM pages 5001-5002 item 1.D, pages 6002-6003 1.C(1-3). |  |  |  | QM / CCS |  |  |
| 1.4.2 | Certify that all CMJC instructions related to ”INSPECTION/CHECK” have been completed in accordance with the CMM. |  |  |  | CCS |  |  |
| 1.5 REPAIR ATTENTION: All used consumable materials and spare parts must be recorded in the Material Used Table and Item Change List. |  |  |  |  |  |  |  |
| 1.5.1 | Perform Repair of the Receptacle Connector. □ Performed □ N/R Ref. CMM page 6003 item 1.C(4). |  |  |  | QM / CCS |  |  |
| 1.5.2 | Certify that all CMJC instructions related to ”REPAIR” have been completed in accordance with the CMM. |  |  |  | CCS |  |  |
| 1.6 ASSEMBLY |  |  |  |  |  |  |  |
| 1.6.1 | Perform Assembly. NOTE: In case of incandescent lamp or autotransformer was removed seal P/N 8000115Y00 (item 1A-65) must be discarded and installed new on. Ref. CMM pages 7002-7008 item 1.C. |  |  |  | QM / CCS |  |  |
| 1.6.2 | Certify that all CMJC instructions related to ”ASSEMBLY” have been completed in accordance with the CMM. |  |  |  | CCS |  |  |
| 1.7 TESTING ATTENTION: All test results must be recorded in the Test / Check Results Table. NOTE: The required values / states and their limits are specified in the Test / Check Results Table. |  |  |  |  |  |  |  |
| 1.7.1 | Make sure that the ambient conditions are suitable for testing. Description; Limit; Acceptability. Temperature; 25 ± 10 °C; □ Ok □ NOT Ok. Relative humidity; Less than 90 %. Atmospheric pressure; 900 to 1100 mbar. Ref. CMM page 1002, items 1.B(3). | QM / CCS |  |  |
| 1.7.2 | Perform Operational Test. - Connect light to the Universal Aircraft Lights Activation Box P/N AT0003.0. - With the "Component" rotary selector select "334711". - Use appropriate touch button for connect and disconnect the circuit a number of times. Ref. CMM page 1002, item 1.D(1). |  |  |  | QM / CCS |  |  |
| 1.7.3 | Perform Insulation Test. Connect connectors pins “A” and “B” together and apply 500 VDC with a Megohmmeter between all the connected pins and a position on the housing assy. Ref. CMM page 1003, item 1.D(2). |  |  |  | QM / CCS |  |  |
| 1.7.4 | Perform Bonding Test. □ Performed □ N/R NOTE: For P/N 4358532 only. With a Milliohmmeter measure the resistance between a position on the housing assy and the ground terminal. Ref. CMM page 1003, item 1.D(3). |  |  |  | QM / CCS |  |  |
| 1.7.5 | Perform Post Test Procedures. Certify that all CMJC instructions related to ”TESTING” have been completed in accordance with the CMM. □ All tests passed □ One or more tests NOT passed Note: If any test fails, mark the Logo Light as unrepairable. |  |  |  | CCS |  |  |
| 1.8 TEST / CHECK RESULTS |  |  |  |  |  |  |  |
| Item | CMJC | Test / Check | Limits | Value |  | Test Result |  |
| 1 | 1.7.2 | Operational Test. Ref. CMM page 1002, item 1.D(1). | Lamp comes "ON". | N/R |  | □ Pass □ Fail |  |
|  |  |  | Current consumption is 2,8 A maximum. |  |  | □ Pass □ Fail |  |
| 2 | 1.7.3 | Insulation Test. Ref. CMM page 1003, item 1.D(2). | For P/N’s 4292491 / 4328733 resistance is more than 20 MΩ. |  |  | □ Pass □ Fail |  |
|  |  |  | For P/N 4358532 resistance is more than 100 MΩ. |  |  |  |  |
| 3 | 1.7.4 | Bonding Test. NOTE: For P/N 4358532 only. Ref. CMM page 1003, item 1.D(3). | Resistance is not more than 10 mΩ under 10 A. |  |  | □ Pass □ Fail □ N/A |  |
| 2. RELEASE RECORDS |  |  |  |  |  |  |  |
| 2.1 | Even shift of technological operations were done during maintenance process, I certify that this is not finally result on component condition at the moment of release to service and not affect its airworthiness. I certify that: - Work performed; - □ Work Package closed □ Work Package NOT closed - A general verification is carried out to ensure that the component is clear of all tools, equipment and any extraneous parts or material; - □ Release Certificate Issued □ Release Certificate NOT Issued |  |  |  | CCS |  |  |

3. WORK DETAILS

| ITEM CHANGE LIST (for Items without s/n) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPL № | P/N |  | Description |  |  | QTY |  | Reason |  |  |  | Detected by Sign/Date |  |
| ITEM CHANGE LIST (for Items with s/n) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IPL № | P/N |  | Description | S/N rem |  | S/N inst |  |  | Reason |  |  | Detected by Sign / Date |  |
| NOTE (completed in case of P/Ns of removed and installed items do not matched) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REPAIR ACTION LIST Filled only if any actions from REPAIR part of CMM was DET- detected, PFMD*-performed and VRF**-verified) * -marked as N/A in case by Contracted MRO performance, * and **- filled by CС/S only |  |  |  |  |  |  |  |  |  |  |  |  |  |
| № | IPL № | Repair Action |  |  | M/H | CMM/SPM Reference |  |  |  | Sign / Date |  |  |  |
|  |  |  |  |  |  |  |  |  |  | DET | PFMD* |  | VRF** |
| ADDITIONAL INSPECTIONS LIST (NDT, battery supplementary tests etc) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| № | Report № |  | Inspection action |  |  | M/H | CMM/SPM Reference |  |  |  |  | Sign / Date |  |
| ADD |  |

| 4. MATERIAL USED (used specification or brand where are several allowable alternatives, should be colored) |  |  |  |
| --- | --- | --- | --- |
| № | Specification | Description | Serviceable Tag No |